SECTION 08 42 33

REVOLVING DOORS

- GENERAL

SECTION INCLUDES

Manual Revolving doors of the following type:

Curved sidewalls, canopy, ceiling, door wing, hardware, glass, drive system and emergency collapsing mechanism as required for installation. (BoonAssist TQM Revolving Door.)

RELATED SECTIONS

Section 07915- Sealant, Caulking and Seals

Section 08400- Entrances and Storefronts

Section 087100- Door Hardware

Section 08810- Glass and Glazing

Section 09600- Flooring

Section 16123- Electrical Supply and Termination

REFERENCES

American National Standards Institute (ANSI):

ANSI/BHMA A156.27 – American National Standard for Power and Manual Operated Pedestrian Doors

ANSI Z97.1 - Safety Glazing Material Used in Buildings.

Fenestration & Glazing Industry Association (FGIA)- previously known as the American Architectural Manufacturers Association (AAMA):

AAMA 611- Voluntary Specification for Anodized Architectural Aluminum

AAMA 2604- Voluntary specification, Performance requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusion and Panels.

AAMA 2605- Voluntary specification, performance requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

American Society for Testing and Materials (ASTM):

ASTM A 36- Standard Specification for Carbon Structural Steel

ASTM A276- Standard Specification for Stainless Steel Bars and Shapes

ASTM A 480/A 480M- Standard Specification for General Requirements for Flat Rolled Stainless and Heat-Resisting Steel Plate, Sheet and Strip

ASTM A666- Standard Specification for Annealed or Cold Worked Austenitic Stainless Steel Sheet, Strip, Plate and Flat Bar.

ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

ASTM B 221- Standard Specification for Aluminum and Aluminum- Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

ASTM C1048- Standard Specification for Heat-Strengthened and Fully Tempered Flat Glass

ASTM C 1172- Standard Specification for Laminated Flat Gass

ASTM E283 - Standard Test Method for Determining the Rate of Air Leakage Through Exterior Windows, Curtain Walls, and Doors Under Specified Pressure Differences Across the Specimen.

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Submit manufacturer's product data sheets, including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Submit manufacturer's shop drawings, including pertinent dimensions, general construction, component connections and locations, anchorage methods and locations, hardware and installation details.

Operation and Maintenance Manual: Submit manufacturer's operation and maintenance manual.

QUALITY ASSURANCE

Manufacturer's Qualifications:

Specialization in the supply of revolving doors with minimum of 10 years’ experience.

Factory-Trained supervisor shall be supplied during installation of the revolving door.

Installer's Qualifications:

Locally located factory-trained field service technician shall be provided to competently service revolving door; and to provide for the local support of the customer’s service technicians in the event the customer’s trained technician is not available. In the event there are no service technicians matching the aforementioned description and requirements, the manufacturer, Boon Edam, Inc., is to be utilized for service.

Local certified Boon Edam, Inc. distributor required if not being installed by the manufacturer, Boon Edam, Inc.

Approved by manufacturer.

Single Source Requirements: To the greatest extent possible provide products specified in this section from a single manufacturer.

DELIVERY, STORAGE, AND HANDLING

Delivery: Deliver materials to site protected from damage.

Storage: Store materials in clean, dry area indoors, off-ground in manufacturer's unopened packaging until ready for installation and in accordance with manufacturer's instructions.

Handling: Protect materials and finish from damage during handling and installation.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

Floor must be dead level at any point within the footprint of the revolving door.

Other trades: General Contractor shall advise of any inadequate conditions or equipment

MAINTENANCE SERVICE

The manufacturer shall offer a dispatch procedure that shall be available 24 hours per day, 365 days per year to facilitate proper service capability.

A manufacturer's designated service contact shall obtain malfunction information and dispatch appropriate service provider to project location.

Submission for maintenance service via Boon Edam, Inc. Phone: 910-984-8094 or us.techsupport@boonedam.com

A geographically assigned installation provider shall be trained and certified to provide maintenance service.

COORDINATION AND SCHEDULING

Conference: Convene a pre-installation conference to establish procedures to maintain optimum working conditions and to coordinate this work with related and adjacent work by Owner.

Coordinate sizes and locations of recesses in concrete if applicable.

Field Measurements: Verify actual dimensions of openings to receive revolving door entrances by field measurements before fabrication and indicate on shop drawings.

WARRANTY

Warranty: Provide manufacturer's standard warranty against defects in materials and workmanship. Warranty shall be one year from date of installation.

Warranty Period: 1 Year (12 Months)

During the warranty period a factory-trained technician shall perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

The Warranty excludes glass breakage, normal wear on finishes or damage that occurs due to abuse, misuse or acts of God.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Boon Edam, Inc. 402 McKinney Parkway, Lillington NC 27546. (910) 814-3800 Fax: (910) 814-3889  Website: www.boonedam.us

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

REVOLVING ENTRANCE DOORS

Revolving Entrance Door: Provide Boon Assist TQM Revolving Door as manufactured by Boon Edam, Inc.

Door Construction:

Curved side walls shall have a standard inside diameter of 6'6" Inside Diameter and be manufactured from six (6) extruded aluminum posts and two (2) extruded bottom rails. the diameter added to the height shall not exceed 17ft per ANSI 156.27.

Canopy shall consist of four (4) one-piece extruded aluminum sections 12"

Provide Three (3) or Four (4) doors wings designed and manufactured of 1 5/8” x 2 5/8” narrow stile aluminum extrusions and reinforced with internal aluminum door corners for strength Door wings must utilize removable genuine horsehair weather stripping on three sides. Door wings must be capable of folding forward or backward allowing for emergency egress.

Ceiling shall be fabricated of formed aluminum sheet in a pie-shaped configuration. Each section must be secured in position and removable only by authorized personnel.

Equipment

Provide BoonAssist drive system permitting for low force automatic operation. Drive system to provide for speed-limiting functionality to prevent increased speed of the door during manual operation.

Provide emergency collapsing system using precision-engineered door hangers and discs permitting for doors wings to be collapsed, folded or stored in book fold position. Hangers to provide tension to hold door sings in position unless strong force to exceed 130 lbs. is applied to ouster stile of the door wing.

Provide two (2) surface applied slide bolt locks 1 ¾” x 5 5/8” with removable, keyed cylinders that lock into the floor on the two (2) interior door wings.

Provide electric shaft fail-safe locking mechanism with two (2) position post-mounted key switch for activation. Electric locking to be designed to prevent rotation of the door wings. Electric locking to be disengaged by loss of power form the fire suppression system. Coordination of a 120-230 VAC power source to drive box required. (Not provided by Boon Edam, Inc.)

Provide electric wing lock mechanism with a two (2) position post-mounted key switch which activates locking. When engaged, the electric locking to prevent the collapsing of the door wings. Electric locking to be disengaged by loss of power form the fire suppression system. Coordination of a 120-230 VAC power source to drive box required from above a minimum 8” canopy. (Not provided by Boon Edam, Inc.)

Provide an approved amount of 3” 110-230VAC 9W LED recessed into the ceiling. Coordination of a 120-230 VAC power source required (Not provided by Boon Edam, Inc.)

Materials:

All flat glass in door wings shall be 1/4” clear tempered safety glass.

All curved glass shall be 7/16” clear tempered safety glass.

- All aluminum/extrusions shall be commercial grade aluminum alloy 6063-T6 per ASTM B-221.

All aluminum sheets to meet ASTM B-209 and be of .063 minimum thicknesses.

Provide genuine horsehair weatherstripping on all required edges of the doors wings to provide seal between door wings and drum that meets ASTM E-283

Provide a rigid rubber-tipped bumper located on the top upper right-hand corner of each door wing designed to protect wings from contacting with one in another when in the book fold position.

Provide floor mounted pivot under the center shaft to provide smooth rotation.

Provide extruded center shaft composed of aluminum alloy 6061-T6 per ASTM B-221 with connections to the speed control and pivot.

Provide one Vertical  (1) extruded aluminum1 ¼” x ½” push bar per door wing finished to match door (1” diameter aluminum, stainless or architectural bronze available as an option)

Provide dust cover made of HDPE (high density polyethylene).  Dust cover to be installed on top of the canopy.

Finish:

Painted Coatings:

- AAMA 2605 Superior Organic Coatings (e.g.: Duranar, Fluoropon, 70% Kynar Fluoropolymers).

- AAMA 2604 High Performance Organic Coatings (e.g. Powder Coating).

- EXECUTION

PREPARATION

Installer must examine the location and advise the Contractor of any site conditions unacceptable for proper installation of product. The minimum conditions necessary to initiate installation are:

Floor must be dead level at any point within the footprint of the door.

Finished floor matwell trim not yet installed.

Power supply (110-240VAC) must be installed.

INSTALLATION

    - Install revolving doors in accordance with manufacturer’s printed instructions. Set unis level, plumb, and with uniform hairline joints. Anchor securely in place. Use only factory trained installers.

FIELD QUALITY CONTROL

Manufacturer's Field Services:

Manufacturer's representative shall provide technical assistance and guidance for installation of the BoonAssist TQM Revolving Door.

ADJUSTING

    - Installer shall adjust door, hardware and sensors for smooth operation and proper performance.

DEMONSTRATION AND TRAINING

    - A factory-trained installer shall demonstrate to the Owner’s maintenance crew the proper operation of the BoonAssist TQM Revolving Door and the necessary service requirements such as lubrication, cleaning and inspection of components upon completion of installation.

CLEANING AND PROTECTION

    - Clean metal and glass surfaces carefully after installation in accordance with manufacturer’s instructions.

END OF SECTION