SECTION 08 42 33

REVOLVING DOORS

- GENERAL

SECTION INCLUDES

All-Glass Manual Revolving doors of the following type:

Curved glass sidewalls, glass ceiling, door wings, hardware, speed control and emergency collapsing mechanism as required for installation. (TQ Crystal Revolving Door.)

RELATED SECTIONS

Section 07915- Sealant, Caulking and Seals

Section 08400- Entrances and Storefronts

Section 087100- Door Hardware

Section 08810- Glass and Glazing

Section 09600- Flooring

REFERENCES

American National Standards Institute (ANSI):

ANSI/BHMA A156.27 – American National Standard for Power and Manual Operated Pedestrian Doors

ANSI Z97.1 - Safety Glazing Material Used in Buildings.

Fenestration & Glazing Industry Association (FGIA)- previously known as the American Architectural Manufacturers Association (AAMA):

AAMA 611- Voluntary Specification for Anodized Architectural Aluminum

AAMA 2604- Voluntary specification, Performance requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusion and Panels.

AAMA 2605- Voluntary specification, performance requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

American Society for Testing and Materials (ASTM):

ASTM A 36- Standard Specification for Carbon Structural Steel

ASTM A276- Standard Specification for Stainless Steel Bars and Shapes

ASTM A 480/A 480M- Standard Specification for General Requirements for Flat Rolled Stainless and Heat-Resisting Steel Plate, Sheet and Strip

ASTM A666- Standard Specification for Annealed or Cold-Worked Austenitic Stainless Steel Sheet, Strip, Plate and Flat Bar.

ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

ASTM B 221- Standard Specification for Aluminum and Aluminum- Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

ASTM C 1048 Standard Specification for Heat-Strengthened and Fully Tempered Flat Glass

ASTM C 1172- Standard Specification for Laminated Flat Glass

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Submit manufacturer's product data sheets, including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Submit manufacturer's shop drawings, including pertinent dimensions, general construction, component connections and locations, anchorage methods and locations, hardware and installation details.

Operation and Maintenance Manual: Submit manufacturer's operation and maintenance manual. Include spare parts list.

QUALITY ASSURANCE

Manufacturer's Qualifications:

Specialization in the supply of revolving doors with a minimum of 5 years’ experience.

Factory-Trained supervisor shall be supplied during installation of the revolving door.

Installer's Qualifications:

Locally located factory-trained field service technician shall be provided to competently service revolving door; and to provide for the local support of the customer’s service technicians in the event the customer’s trained technician is not available. In the event there are no service technicians matching the aforementioned description and requirements, the manufacturer, Boon Edam, Inc., is to be utilized for service.

Local certified Boon Edam, Inc. distributor required if not being installed by the manufacturer, Boon Edam, Inc.

Approved by manufacturer.

Single Source Requirements: To the greatest extent possible provide products specified in this section from a single manufacturer.

DELIVERY, STORAGE, AND HANDLING

Delivery: Deliver materials to site protected from damage.

Storage: Store materials in clean, dry area indoors, off-ground in manufacturer's unopened packaging until ready for installation and in accordance with manufacturer's instructions.

Handling: Protect materials and finish from damage during handling and installation.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

TQ Crystal Revolving Door installs on dead level finished floor.

Floor must be dead level at any point within the footprint of the revolving door.

Other trades: General Contractor shall advise of any inadequate conditions or equipment

MAINTENANCE SERVICE

The manufacturer shall offer a dispatch procedure that shall be available 24 hours per day, 365 days per year to facilitate proper service capability.

A manufacturer's designated service contact shall obtain malfunction information and dispatch appropriate service provider to project location.

Submission for maintenance service via Boon Edam, Inc. Phone: 910-984-8094 or us.techsupport@boonedam.com

A geographically assigned installation provider shall be trained and certified to provide maintenance service.

COORDINATION AND SCHEDULING

Conference: Convene a pre-installation conference to establish procedures to maintain optimum working conditions and to coordinate this work with related and adjacent work as required by Owner.

Coordinate sizes and locations of recesses in concrete if applicable.

Field Measurements: Installer to verify actual dimensions of openings to receive revolving door entrances by field measurements before fabrication and indicate on shop drawings.

WARRANTY

Warranty: Provide manufacturer's standard warranty against defects in materials and workmanship. Warranty shall be one year from date of shipping date.

Warranty Period:  1 Year (12 Months)

During the warranty period a factory-trained technician shall perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

The Warranty excludes glass breakage, normal wear on finishes or damage that occurs due to abuse, misuse or acts of God.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Boon Edam, Inc. 402 McKinney Parkway, Lillington NC 27546. (910) 814-3800 Fax: (910) 814-3889  Website: www.boonedam.us

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

REVOLVING ENTRANCE DOORS

Revolving Entrance Door: Provide TQ Crystal Revolving Door as manufactured by Boon Edam, Inc.

Door Construction:

Curved side walls shall have a standard inside diameter of 6-6” (only available on 4-wing), 7-0”, 7’6”, 8’0” and be manufactured from 11/16th curved laminated safety glass

Three or Four door wings as designed and manufactured from ½” tempered safety glass with stainless steel clad polycarbonate channels and rubber with felt contact weather stripping on three sides. Door wings must be capable of folding forward or backward allowing for emergency egress.

Provide doorwing type as follows: Standard Stainless Steel Glass Clamps Choose an item.

Ceiling shall be fabricated from 11/16” tempered laminated safety glass.

Provide polished stainless-steel connectors between sidewalls and ceiling.

Equipment

Provide floor-mounted manual speed control attaches to the center rotating shaft and limits the rotation speed of the door to a preset RPM not to exceed 12 revolutions per minute. Operating in an oil bath, the centrifugal braking unit can be adjusted for lower maximum speeds to meet code requirements.

Provide emergency collapsing system using precision-engineered door hangers and discs permitting for doors wings to be collapsed, folded or stored in book fold position. Hangers to provide tension to hold door wings in position unless strong force to exceed 130 lbs. is applied to outer stile of the door wing.

Provide electric shaft fail-safe locking mechanism with two (2) position post-mounted key switch for activation. Electric locking to be designed to prevent rotation of the door wings. Electric locking to be disengaged by loss of power form the fire suppression system. Coordination of a 120-230 VAC power source to drive box required. (Not provided by Boon Edam, Inc.)

Provide electric wing lock mechanism with a two (2) position post-mounted key switch which activates locking. When engaged, the electric locking to prevent the collapsing of the door wings. Electric locking to be disengaged by loss of power form the fire suppression system. Coordination of a 120-230 VAC power source to drive box required from above a minimum 8” canopy. (Not provided by Boon Edam, Inc.)

Provide two (2) standard 1 3/4” x 5 5/8” surface-mounted deadbolt locks with removable, keyed cylinders that lock into the floor on the two interior door wings.

Materials:

All flat glass in door wings shall be ½” clear tempered safety glass.

All curved glass in sidewalls shall be clear 11/16” curved laminated safety glass.

All flat ceiling glass shall be 11/16” tempered laminated safety glass.

Provide weatherstripping composed of rubber with felt contact strip at sidewalls and ceiling. Rubber at floor. Weatherstripping required on all edges of door wings to provide a seal between doors wings and drum that meets or exceeds ASTM E-283 standards for air infiltration

Provide a rigid rubber-tipped bumper located on the top upper right-hand corner of each door wing designed to protect wings from contacting with one in another when in the book fold position.

Provide ceiling mounted pivot above the center shaft to provide for smooth rotation.

Provide stainless steel center shaft with connection to the speed control and pivot

Provide 1” diameter vertical stainless steel push bars.

Provide square stainless steel floor mounted cover over recessed speed

Provide stainless steel embed frame (Matwell Ring) required to be installed in the floor prior to door installation.

- EXECUTION

PREPARATION

Installer must examine the location and advise the Contractor of any site conditions unacceptable for proper installation of product. The minimum conditions necessary to initiate installation are:

Floor must be dead level at any point within the footprint of the door.

Finished floor matwell trim not yet installed.

Power supply (110-240VAC) must be installed if required for locking options.

INSTALLATION

    - Install revolving doors in accordance with manufacturer’s printed instructions. Set unis level, plumb, and with uniform hairline joints. Anchor securely in place. Use only factory trained installers.

FIELD QUALITY CONTROL

Manufacturer's Field Services:

Manufacturer's representative shall provide technical assistance and guidance for installation of the TQ Crystal Revolving Door.

ADJUSTING

    - Installer shall adjust door, hardware and sensors for smooth operation and proper performance.

DEMONSTRATION AND TRAINING

    - A factory-trained installer shall demonstrate to the Owner’s maintenance crew the proper operation of the Crystal TQ Revolving Door and the necessary service requirements such as lubrication, cleaning and inspection of components upon completion of installation.

CLEANING AND PROTECTION

    - Clean metal and glass surfaces carefully after installation in accordance with manufacturer’s instructions.

END OF SECTION