-

SECTION 08 42 33

SECURITY REVOLVING DOORS

- GENERAL

SECTION INCLUDES

Security Revolving doors of the following type, Tourlock 180+90 Security Revolving Door:

Curved side walls, canopy, ceiling, LED ceiling lights, hardware, glass, motor drive system, emergency collapsing mechanism, communication system, self-diagnostics tool, safety system and infrared sensor system as required for installation.

RELATED SECTIONS

Section 07915- Sealant, Caulking and Seals

Section 08400- Entrances and Storefronts

Section 087100- Door Hardware

Section 08810- Glass and Glazing

Section 09600- Flooring

Section 16123- Electrical Supply and Termination

REFERENCES

American National Standards Institute (ANSI):

ANSI Z97.1 - Safety Glazing Material Used in Buildings.

ANSI 156.27 - Power and Manual Operated Revolving Pedestrian Doors

Fenestration & Glazing Industry Association (FGIA)- previously known as the American Architectural Manufacturers Association (AAMA):

AAMA 611- Voluntary Specification for Anodized Architectural Aluminum

AAMA 2604- Voluntary specification, Performance requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusion and Panels.

AAMA 2605- Voluntary specification, performance requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

American Society for Testing and Materials (ASTM):

ASTM A 480/A 480M- Standard Specification for General Requirements for Flat Rolled Stainless and Heat-Resisting Steel Plate, Sheet and Strip

ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

ASTM B 221- Standard Specification for Aluminum and Aluminum- Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

Canadian Standards Association. (CSA Group)- CSA22.2#247

Underwriters Laboratories Inc. (UL)- UL Standard 325 and UL 2593

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Submit manufacturer's product data sheets, including installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Submit manufacturer's shop drawings, including pertinent dimensions, general construction, component connections and locations, anchorage methods and locations, sensors, hardware and installation details.

QUALITY ASSURANCE

Manufacturer's Qualifications:

Specialization in the supply of automatic security revolving doors with a minimum of 10 years’ experience.

Factory-Trained supervisor shall be supplied during installation of the automatic security revolving door.

Installer's Qualifications:

Locally located factory-trained field service technician shall be provided to competently service the automatic security revolving door; and to provide for the local support of the customer’s service technicians in the event the customer’s trained technician is not available. In the event there are no service technicians matching the aforementioned description and requirements, the manufacturer, Boon Edam, Inc., is to be utilized for service.

Local certified Boon Edam, Inc. distributor required if not being installed by the manufacturer, Boon Edam, Inc.

Approved by manufacturer.

Single Source Requirements: To the greatest extent possible provide products specified in this section from a single manufacturer.

DELIVERY, STORAGE, AND HANDLING

Delivery: Deliver materials to site protected from damage.

Storage: Store materials in clean, dry area indoors, off-ground in manufacturer's unopened packaging until ready for installation and in accordance with manufacturer's instructions.

Handling: Protect materials and finish from damage during handling and installation.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

Tourlock 180 installs on finished floor only.

Floor must be dead level within 1/8” at any point within the footprint of the revolving door.

MAINTENANCE SERVICE

The manufacturer shall offer a dispatch procedure that shall be available 24 hours per day, 365 days per year to facilitate proper service capability.

A manufacturer's designated service contact shall obtain malfunction information and dispatch appropriate service provider to project location.

Toll free phone number, 1-877-237-2687, shall be prominently displayed on header of each operator.

A geographically assigned installation provider shall be trained and certified to provide maintenance service.

COORDINATION AND SCHEDULING

Conference: Convene a pre-installation conference to establish procedures to maintain optimum working conditions and to coordinate this work with related and adjacent work.

Coordinate sizes and locations of recesses in concrete if applicable.

Field Measurements: Verify actual dimensions of openings to receive revolving door entrances by field measurements before fabrication and indicate on shop drawings.

WARRANTY

Warranty: Provide manufacturer's standard warranty against defects in materials and workmanship. Warranty shall be one year from date of installation.

Warranty Period: 1 Year (12 Months)

During the warranty period a factory-trained technician shall perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

The Warranty excludes glass breakage, normal wear on finishes or damage that occurs due to abuse, misuse or acts of God.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Boon Edam, Inc. 402 McKinney Parkway, Lillington NC 27546. (910) 814-3800 Fax: (910) 814-3889  Website: www.boonedam.us

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

SECURITY REVOLVING DOOR

Security Revolving Door: Tourlock 180+90 designed Single Direction Stereovision (SD) and Dual Direction Stereovision (DD) as manufactured by Boon Edam, Inc.

Provide side walls and canopy constructed of a standard inside diameter of 6'6" and to be manufactured from six (6) extruded aluminum posts and (4) piece aluminum canopy and four (4) extruded aluminum bottom rails.

Height Under Canopy 7'0"

Overall Height 8'0"

Segmented sidewalls are not acceptable due to premature elopement failure, compromised security, excessive noise and visual appearance.

Provide Narrow Stile Door Wings- Four (4) wings as designed and manufactured of 1 5/8” x 2 5/8” narrow stile aluminum extrusions and reinforced with internal aluminum door corners for strength.  Door wings must utilize removable horsehair weather stripping on three sides. Door wings must be capable of book-folding forward or backward allowing for emergency egress.

Ceiling shall be fabricated of formed aluminum sheet in a pie-shaped configuration.  The ceiling system must fasten and secure each section in a secured position and can only be removed by authorized personnel.  Servicing of the doors and access to the controls must be from the floor position. Doors that require servicing access from the top are not acceptable. Ceiling pies to be key lockable with tamper switch installed to be monitored by access control.

Performance/ Throughput Requirements:

General: The revolving door unit must be a 4-wing SIMULTANEOUS TWO-WAY TRAFFIC door.  Throughput is defined as the number of people per minute which can pass through a door in one direction only.

Throughput is a function of the number of compartments in the door multiplied by the number of full revolutions a door can safely make in a one-minute time frame.  Given that the number of door compartments is four (4) and the safe maximum number of revolutions per minute is six (6), the maximum throughput in any one direction is 24 people per minute.  Taking into consideration all user and environmental factors, the average expected throughput in any one direction is between 15 and 20 people per minute.

Active/Passive Piggyback detection.  For throughput enhancement, the door shall be capable of selective control which allows switching between actively blocking and rejecting attempted piggyback attempts, or passive detection with allowed passage and alarm notification to Security of a potential piggyback attempt.   Selective control is managed through the Access Control System.

Finish:

- Finishes to be available for the enclosure walls, rotating door wings and ceiling.

Powder Coatings to be AAMA 2604 High performance Organic Coatings.

Components:

All flat glass in door wings to be clear safety glass and all curved glass to be clear bent safety glass.

All commercial grade extrusions to be of aluminum alloy 6063- T6 per ASTM B-221.

Genuine horsehair weather stripping to be installed all required edges of doors wings to provide a seal between each of the door wings and drum that meets ASTM-E-283.

All glass to be sealed with push in glazing vinyl.

Floor mounted pivot to be under the central shaft to provide smooth rotation.

SEQUENCE OF OPERATION

Sequence of Operation: The sequence of operation is critical to the management of false rejections and mitigating risks of false acceptances. The specification requires the exact sequence of operation to be incorporated into the design.

In-Bound Sequence (Including Stereovision functionality): Authorization device located at door entrance (not provided by Boon Edam, Inc.)

Credential authorization required to activate authorization sequence of door.

Upon authorization signal, door visual indicators will change from RED to GREEN coinciding with an audible alert. Alert is designed to indicate authorization is approved.

Person steps into the compartment Infrared Start-Up sensors (IRS) detect the person’s presence. Upon detection, the door will start to rotate.

If a person does not step into the presence of the IRS the door will time-out and reset to Standby mode (i.e. Green Indicator will change to Red)

At 90 degrees of rotation, the Stereovision system will perform the following analysis to determine whether zero persons, one person, more persons, or suspicious activity is detected in the compartment.

Zero Persons: If there is no presence detected by Stereovision, the door system will rotate an additional 90 degrees and rest in the standby mode. (Plus 90 feature may be activated allowing authorized passage in the following compartment)

One Person: If one person is detected, the door will continue to rotate an additional 90 degrees and rest in the standby mode (Plus 90 feature may be activated allowing authorized passage in the following compartment)

More Persons: If multiple persons are detected, the door will block at 90 degrees of rotation to prevent passage.

Suspicious Activity: If suspicious activity is detected the door will block at 90 degrees of rotation to prevent passage.

- (If More or Suspicious persons are in the compartment at 90 degrees of rotation, the Sterovision system will send a signal to the PLC of the condition and the door will stop rotation and block entry.  An audible alarm will sound, and visual RED indicator will flash.  The door will then reverse rotation to the (+) blocking position, and allow the unauthorized person(s) to exit.  Once the person(s) have exited, the door will reset to the standby rest position (X).

Out-Bound Sequence of Operation (Including Stereovision functionality): Authorization device located at door entrance (Not provided by Boon Edam, Inc.)

Credential authorization required to activate authorization sequence of door.

Upon authorization signal, door visual indicators will change from RED to Green along with an audible alert to indicate authorization is approved.

Person steps into the compartment, and Infrared Start-Up sensors (IRS) detect person’s presence. Upon detection, the door will rotate.

If a person does not step into the presence of the IRS the door will time out and reset to standby mode (i.e. Green indicator will change back to red)

At 90 degrees of rotation, the Stereovision system will perform the following analysis to determine whether zero persons, one person, more persons or suspicious activity is detected in the compartment.

Zero Persons: If there is no presence detected by Stereovision the door system will rotate an additional 90 degrees and rest in standby mode. (Plus 90 feature may be activated allowing authorized passage in the following compartment)

One Person: If one person is detected the door will continue to rotate an additional 90 degrees and rest in the standby mode (Plus 90 feature may be activated allowing authorized passage in the following compartment)

More Persons: If multiple persons are detected the door will block at 90 degrees of rotation to prevent passage.

Suspicious Activity: If suspicious activity is detected the door will block at 90 degrees of rotation to prevent passage.

- (If More or Suspicious persons are in the compartment at 90 degrees of rotation, the Sterovision system will send a signal to the PLC of the condition and the door will stop rotation and block entry.  An audible alarm will sound, and visual RED indicator will flash.  The door will then reverse rotation to the (+) blocking position, and allow the unauthorized person(s) to exit.  Once the person(s) have exited, the door will reset to the standby rest position (X).

- EXECUTION

PREPARATION

Installer must examine the location and advise the Contractor of any site conditions unacceptable for proper installation of product. The minimum conditions necessary to initiate installation are:

Floor must be dead level at any point within the footprint of the door.

Finished floor must be installed.

Exterior grade for adequate drainage must be properly designed.

Power supply (110-240VAC) must be installed.

INSTALLATION

    - Install revolving doors in accordance with manufacturer’s printed instructions. Set unis level, plumb, and with uniform hairline joints. Anchor securely in place. Use only factory trained installers.

FIELD QUALITY CONTROL

Manufacturer's Field Services:

Manufacturer's representative shall provide technical assistance and guidance for installation of the Tourlock 180+90 Security Revolving Door.

Before placing security revolving door into operation, technician shall inspect and approve doors for compliance with ANSI/BHMA 156.27 Certified technician shall be approved by manufacturer.

ADJUSTING

    - Installer shall adjust door, hardware and sensors for smooth operation and proper performance.

DEMONSTRATION AND TRAINING

    - A factory-trained installer shall demonstrate to the Owner’s maintenance crew the proper operation of the Tourlock 180+90 and the necessary service requirements such as lubrication, cleaning and inspection of components upon completion of installation.

CLEANING AND PROTECTION

Clean metal and glass surfaces carefully after installation as per the manufacturer’s directions

END OF SECTION