SECTION 11-14-00

PEDESTRIAN CONTROL EQUIPMENT

- GENERAL

SECTION INCLUDES

Security Optical Turnstiles of the following type:

Cabinet, swinging glass panels, motor columns with drive system, hardware cabling, self-diagnostics tool, safety system and infrared system. (Speedlane Swing)

RELATED SECTIONS

Section 09600- Flooring

Section 11 14- Pedestrian Control Equipment

Section 11 14 13.19- Turnstiles

Section 11 14.53- Pedestrian Security Equipment

Section 16123- Electrical Supply and Termination

Section 26056- Grounding and Bonding for Electrical Systems

Section 28100- Electronic Access Control and Intrusion Detection

Section 281300- Access Control

Section 281400- Access Control System Hardware

Section 281600- Intrusion Detection

REFERENCES

American National Standards Institute (ANSI):

ANSI Z97.1 - Safety Glazing Material Used in Buildings.

Fenestration & Glazing Industry Association (FGIA)- previously known as the American Architectural Manufacturers Association (AAMA):

AAMA 2604- Voluntary specification, Performance requirements and Test Procedures for High Performance Organic Coatings on Aluminum Extrusion and Panels.

AAMA 2605- Voluntary specification, performance requirements and Test Procedures for Superior Performing Organic Coatings on Aluminum Extrusions and Panels.

American Society for Testing and Materials (ASTM):

ASTM A 480/A 480M- Standard Specification for General Requirements for Flat Rolled Stainless and Heat-Resisting Steel Plate, Sheet and Strip

ASTM B 209 – Standard Specification for Aluminum and Aluminum-Alloy Sheet and Plate

ASTM B 221- Standard Specification for Aluminum and Aluminum- Alloy Extruded Bars, Rods, Wire, Profiles, and Tubes.

CSA Group (Canada)

CSA22.2 #247

Underwriters Laboratories Inc. (UL).

UL Standard 325

UL Standard 2593

SUBMITTALS

Submit under provisions of Section 01 30 00 - Administrative Requirements.

Product Data: Submit manufacturer's product data sheets, along with installation details, material descriptions, dimensions of individual components and profiles, fabrication, operational descriptions and finishes:

Preparation instructions and recommendations.

Storage and handling requirements and recommendations.

Installation methods.

Shop Drawings: Submit manufacturer's shop drawings, including pertinent dimensions, general construction, component connections and locations, anchorage methods and locations, safety plus sensors, hardware and installation details.

QUALITY ASSURANCE

Manufacturer's Qualifications:

Specialization in the supply of security optical turnstiles with a minimum of 10 years’ experience.

Factory-Trained supervisor shall be supplied during installation of the security optical turnstile.

Installer's Qualifications:

Local factory-trained field service technician shall be provided to competently service the security turnstile; and to provide for the local support of the customer’s service technicians in the event the customer’s trained technician is not available. In the event there are no service technicians matching the aforementioned description and requirements, the manufacturer, Boon Edam, Inc., is to be utilized for service.

Local certified Boon Edam, Inc. distributor required if not being installed by the manufacturer, Boon Edam, Inc.

Approved by manufacturer.

Single Source Requirements: To the greatest extent possible provide products specified in this section from a single manufacturer.

DELIVERY, STORAGE, AND HANDLING

Delivery: Deliver materials to site protected from damage.

Storage: Store materials in clean, dry area indoors, off-ground in manufacturer's unopened packaging until ready for installation and in accordance with manufacturer's instructions.

Handling: Protect materials and finish from damage during handling and installation.

PROJECT CONDITIONS

Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

The Speedlane Swing Security Optical Turnstile installs on finished floor only.

Floor must be leveled within 1/8” (2 mm) at any point within the footprint of the Speedlane Swing array.

Floor must be complete with conduit supplied to meet manufacturer’s specified drawings.

MAINTENANCE SERVICE

The manufacturer shall offer a dispatch procedure that will be available 24 hours per day, 365 days per year to facilitate proper service capability.

A manufacturer's designated service contact shall obtain malfunction information and dispatch appropriate service provider to project location.

Submission for maintenance service via Boon Edam, Inc. Phone: 910-984-8094 or us.techsupport@boonedam.com

A geographically assigned installation provider shall be trained and certified to provide maintenance service.

COORDINATION AND SCHEDULING

Conference: Convene a pre-installation conference to establish procedures to maintain optimum working conditions and to coordinate this work with related and adjacent work per Owner request.

Coordinate sizes and locations of recesses in concrete if applicable.

Field Measurements: Verify actual dimensions for installation of optical turnstiles by field measurements before fabrication and indicate on shop drawings.

WARRANTY

Warranty: Provide manufacturer's standard warranty against defects in materials and workmanship. Warranty shall be one year from date of installation.

Warranty Period: 1 Year (12 Months)

During the warranty period a factory-trained technician shall perform service and affect repairs. A safety inspection shall be performed after each adjustment or repair and a completed inspection form shall be submitted to the Owner.

The Warranty excludes glass breakage, normal wear on finishes or damage that occurs due to abuse, misuse or acts of God.

- PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Boon Edam, Inc. 402 McKinney Parkway, Lillington NC 27546. (910) 814-3800 Fax: (910) 814-3889 Website: www.boonedam.us

Substitutions: Not permitted.

Requests for substitutions will be considered in accordance with provisions of Section 01 60 00 - Product Requirements.

SECURITY OPTICAL TURNSTILE

Security Optical Turnstile: Speedlane Swing Security Optical Turnstile constructed with stainless steel cabinets with swinging glass panels as manufactured by Boon Edam, Inc.

Provide 304 stainless steel cabinet panels in #4 brushed stainless steel.

Provide swinging glass panels manufactured of 10 mm flat, clear tempered safety glass. Waist height glass to be at a height of 37"

Provide Top Plate manufactured from Black Safety Glass

Provide Front Cover Plate manufactured from Black Safety Glass

Provide Motor Column and Plinth (base covers) in Powder coated Stainless Steel

Control System housing microprocessor to be installed in a controlled Closed Area

Performance/ Throughput Requirements:

General: Provide security optical turnstiles that have been designed to provide two-way traffic, one-way traffic or a blocked glass panel (closed entry/exit). Throughput is defined as the number of people per minute which can pass through an optical turnstile in one direction only. Average throughput shall be approximately 25-30 people per minute dependent on configuration of access control system.

Finish:

- Finishes to be available for the removable panels, bottom plinth covers, face and top cover plate.

Painted Coatings to be AAMA 2605 Performing Organic Coatings (e.g.: Duranar, Fluropon; 70% Kynar Fluropolymers)

Powder Coatings to be AAMA 2604 High performance Organic Coatings.

Stainless steel components to be # 4 Brushed Satin for cabinet.

Components:

- 1.  All sheet metal shall be 304 stainless steel in the #4 brushed steel or powder coated steel as specified.

- 2. All wing panel glass shall be 10mm clear safety glass. Cabinet panel glass shall be 8 mm clear safety glass. All cabinet top and front glass shall be ¼” black coated safety glass.

- 3. All plate aluminum shall be 6mm 1050-H24 or equivalent plate.

- 4. All hardware to be metric thread to conform to International Organization for Standardization (ISO) or German Institute for Standardization or (DIN) standard.

- 5. Provide tamper switch to be installed on vertical plinth on the secure side of optical turnstile cabinet.

- 6. Provide interior cabinet mounted battery back-up.

- 6. All plastic components to be Dark Grey infrared (IR) polycarbonate.

- Actuation and Safety Control Devices:

Actuation Devices: Wing panel glass actuation by credential reader mounted Under the Reader Lens (Not supplied by Boon Edam, Inc.)

Actuation: Although a component of the turnstile, actuation devices are provided by the Access Control Integrator.

Sensor System:

Entry Sensors designed to detect the initial presence of the user, once the person has passed the entry sensors, any following unauthorized users must be granted access, otherwise a tailgate detection will occur resulting in an alarm.

Tailgate Sensor designed to detect the presence of an individual who has passed through the entry sensors and is moving into the safety sensor area. In conjunction with the Entry sensors, the unit can detect directional movement and indicate potential unauthorized/tailgating entry attempts.

Return Signal Booking Sensors designed to tell whether an authorized user has passed through the unit. A signal is generated from the controller to the Access Control System to verify the user successfully passed into the authorized direction of travel.

Passage Position Sensors designed to determine the position of the user and start the closing sequence of the glass panels. Once a user has passed through the Safety Sensors and clears the Passage Position Sensors, the glass panels will start closing.

Trolley Detection Sensors designed to indicate that a trolley/suitcase has entered with an authorized user. When activated, the lane will remain open an extended period of time to allow for proper passage of the trolley/suitcase.

Climb over Sensor system designed integrated into the top plate to detect persons attempting to climb over the cabinet while the lane is sitting in sleep or standby mode.

Preset the torque parameter with the programming of the drive system to provide for minimal rotation force. Minimizing rotation force permits the door set to be stopped manually by applying pressure against its rotation.

- EXECUTION

PREPARATION

Installer must examine the location and advise the Contractor of any site conditions unacceptable for proper installation of product. The minimum conditions necessary to initiate installation are:

Floor must be dead level within 1/8” at any point within the footprint of the door.

Finished floor must be installed.

Floor must be complete with conduit supplied to meet manufacturer’s specified drawings.

Power supply (110-240VAC) must be installed. Power and communication from floor to the secure side of the cabinet as per approved specified drawings.

INSTALLATION

    - Install turnstile in accordance with manufacturer’s printed instructions. Set unis level, plumb, and with uniform hairline joints. Anchor securely in place. Use only factory trained installers.

FIELD QUALITY CONTROL

Manufacturer's Field Services:

Manufacturer's representative shall provide technical assistance and guidance for installation of Speedlane Swing Security Optical Turnstile

Before placing security optical turnstiles in operation, technician shall inspect and approve swinging glass panels for compliance with ANSI/BHMA Z97.1. Certified technician shall be approved by manufacturer.

ADJUSTING

    - Installer shall adjust turnstile, hardware and sensors for smooth operation and proper performance.

    - Installer to confirm coordination with contractors responsible for all required connections including access control and fire alarm.

DEMONSTRATION AND TRAINING

    - A factory-trained installer shall demonstrate to the Owner’s maintenance crew the proper operation of the Speedlane Swing Security Optical Turnstile and the necessary service requirements such as lubrication, cleaning and inspection of components upon completion of installation.

CLEANING AND PROTECTION

    - Clean metal and glass surfaces carefully using manufacturer’s suggested cleaning materials.

END OF SECTION